Annual Drinking Water Quality Report for 2019
Livingston County Water & Sewer Authority 

1997 D’Angelo Drive, Lakeville, New York 14480

www.co.livingston.state.ny.us/lcwsa.htm

Federal ID Numbers
LCWSA District:        
           
NY2501019

Village of Livonia District         
NY2501022
Town of Geneseo District 3
NY2530023
Town of Geneseo District 2 (N)  NY2530005
Introductiontc \l4 "Introduction
To comply with State and Federal regulations, the Livingston County Water & Sewer Authority (LCWSA) annually issues a report describing the quality of your drinking water.  The purpose of this report is to raise your understanding of drinking water and awareness of the need to protect our drinking water sources. Last year, we conducted over 161 tests for bacteriological contamination. This report provides an overview of last year’s water quality.  Included are details about where your water comes from, what it contains, and how it compares to State standards.  

If you have any questions about this report or concerning your drinking water, please contact Mark Kosakowski, Director of Operations, at (585) 346-3523. We want you to be informed about your drinking water.  If you want to learn more, please attend any of our regularly scheduled LCWSA board meetings.  The meeting times, dates and locations can be obtained by calling the LCWSA office at (585) 346-3523 or on our website at:
https://livingstoncounty.us/DocumentCenter/View/8973/2020-Meeting-Schedule
Where does our water come from?
In general, the sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally occurring minerals and can pick up substances resulting from the presence of animals or from human activities.  Contaminants that may be present in source water include: microbial contaminants; inorganic contaminants; pesticides and herbicides; organic chemical contaminants; and radioactive contaminants.  In order to ensure that tap water is safe to drink, the State and the EPA prescribe regulations which limit the amount of certain contaminants in water provided by public water systems.  The State Health Department and the FDA regulations establish limits for contaminants in bottled water that must provide the same protection for public health.

The LCWSA water supply is obtained from the City of Rochester and is delivered through a connection to the City transmission main located just north of Big Tree Road in Hemlock.  This water originates in the Hemlock and Canadice Lakes watersheds and is treated at the City of Rochester’s Hemlock Lake Water Treatment Facility located on Rix Hill Road in Hemlock.  The water is treated at the Hemlock Lake Plant using coagulation, filtration, disinfection, and fluoridation.  The City of Rochester’s 2019 Annual Water Quality Report is available on-line at:  https://www.cityofrochester.gov/waterquality/ and is also posted on the LCWSA’s website at: https://www.livingstoncounty.us/550/Operations-Development  
In addition, LCWSA re-chlorinates in South Livonia, Lakeville, Conesus, Maple Beach Pump Station, Groveland Station Lower Tank, East Lake Road and Scottsburg Tank to maintain adequate disinfection, and free chlorine residuals throughout the entire distribution system. Water consumed by the Town of Geneseo Water District 3 is purchased from LCWSA (see attached report).  The Village of Geneseo also purchases water wholesale from LCWSA, which is being supplied to the Village from a master meter pit located on West Lake Road in the vicinity of the Livonia/Geneseo Town Line.  The Village in turn supplies this water to a portion of the Town of Geneseo Water District 2, located between the Town Line and the Village’s Water Treatment Plant, on Blue Heron Drive is included in this report.
During 2019, our system did not experience any restriction of our water source.  
Facts and Figures
The Consolidated water system serves a population of about 8315 people through 3326 service connections.  The Livonia Village water system serves a population of about 1305 people through 522 service connections. 
The total water purchased in 2019 from the City of Rochester was approximately 327,405,000 gallons, which is broken down as follows: 311,621,000 gallons for LCWSA District & Village of Livonia District, 813,000 gallons for Town of Geneseo Water District 3 and 14,971,000 gallons for (portion of) Town of Geneseo Water District 2.  The amount of water delivered to LCWSA customers (LCWSA District & Village of Livonia District) was approximately 188,625,000 gallons. This leaves an unaccounted for total of 138,780,000 gallons (42 % of the total amount purchased).  This water was used to flush hydrants, fight fires and for fire drills, and normal flushing of mains.  All other unaccounted for water was a result of: four (4) water main breaks, slowed retail meters, un-metered water, and leakage. Over 257 retail meters were replaced throughout all service areas, as a result of our meter replacement program.  In 2019, most water customers were charged $3.50 per 1,000 gallons of water with an annual minimum water charge per connection of $148 per year.
Are there contaminants in our drinking water?
As the State regulations require, the LCWSA routinely tests your drinking water for Total Coliform Bacteria, Asbestos and Disinfection By-products (Total Trihalomethanes – TTHM, Haloacetic Acids – HAA).
In 2019, the LCWSA took 161 Total Coliform Bacteria samples in the Consolidated water system.  On August 14, 2019 a sample was taken at the Dennison Rd tank and found to have Total Coliform. Additional samples were taken on August 20, 2019 and August 21, 2019 the sample results came back negative for Total Coliform. This was not a violation.
In 2019, the LCWSA took 24 Total Coliform Bacteria samples in the Village of Livonia District. Of these samples, 0 tested positive for coliform bacteria. 
In February, May, August, and November 2019, the LCWSA violated the MCL for TTHM levels at the Groveland Station Upper Tank (7161 Groveland Hill Rd) sample location. On June 18, 2019, the United States Environmental Protection Agency(EPA)  issued an  Administrative Oder for the exceedances. LCWSA is working with our engineers to put together a corrective action plan to correct these exceedances. 
It should be noted that all drinking water, including bottled drinking water, may be reasonably expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the EPA Safe Drinking Water Hotline (800-426-4791) or the Livingston County Health Department at (585) 243-7280.  

	Table of Detected Contaminants

	Contaminant


	Violation
Yes/No

	Date of Sample
	Level Detected
	Unit
Measurement
	MCLG
	Regulatory Limit (MCL, TT or AL)
	Likely Source of Contamination

	Microbiological Contaminants

	Total Coliform1
	no

	8/14/2019
8/15/2019


	2 positive samples
	N/A
	0
	TT=2 or more positive samples in 1 month
	Naturally present in the environment

	Chlorine Residual
	No
	Daily (Entry Point – City of Rochester water supply)
Monthly (Measured throughout distribution system)
	Range
(0.4 -1.5)

Range

(_____)
	mg/l
	N/A
	MRDL= 4.0
	Water additive to control Microbes


	Stage 2 Disinfection Byproducts – LCWSA Consolidated District2

	Contaminant
	Violation

Yes/No

	Date of Samples
	Level Detected

(Range)
	Unit
Measurement
	MCLG
	Regulatory Limit (MCL, TT or AL)
	Likely Source of Contamination

	TTHM - Site 1

7161 Groveland Hill Rd
	Yes
	5/9/18

8/17/18

11/14/18
2/20/19

5/15/19

8/15/19

11/15/19
	86.253
(66-106)


	ug/L
	N/A
 
	80.0

 
	Byproduct of drinking water chlorination needed to kill harmful organisms.  TTHMs are formed when source water contains organic matter.


	TTHM – Site 2

6170 East Lake Rd
	No
	5/9/18

8/17/18

11/14/18
2/20/19

5/15/19

8/15/19

11/15/19
	         71.25

(53-110)


	
	
	
	

	HAA – Site 1

7161 Groveland Hill Rd


	No
	5/9/18

8/17/18

11/14/18
2/20/19

5/15/19

8/15/19

11/15/19
	27

(17-30)
	ug/L
	N/A
 
	60.0

 
	Byproduct of drinking water disinfection needed to kill harmful organisms.

	HAA – Site 2

Site 2

6170 East Lake Rd


	No
	5/9/18

8/17/18

11/14/18
2/20/19

5/15/19

8/15/19

11/15/19
	24.5
(16-37)


	
	
	
	

	Stage 2 Disinfection Byproducts – Village of Livonia Sites2

	TTHM – Site 1

Livonia Fire Department


	No
	5/9/18

8/17/18

11/14/18
2/20/19

5/15/19

8/15/19

11/15/19
	67.25
(52-90)


	ug/L
	N/A
 
	80.0

 
	Byproduct of drinking water chlorination needed to kill harmful organisms.  TTHMs are formed when source water contains organic matter.



	HAA – Site 2 

51 Washington St.

	No
	5/9/18

8/17/18

11/14/18
2/20/19

5/15/19

8/15/19

11/15/19
	22

(15-26)


	ug/L
	N/A
 
	60.0

 
	Byproduct of drinking water disinfection needed to kill harmful organisms.


	Contaminant
	Date of Sample
	Detection

Level

Based on

90th Percentile

Sample
	Unit Measured
	AL
	Likely Source

	Lead5
	9/24/18 
	 7.6
(ND–63)
	ug/L
	15
	Corrosion of household plumbing

	Copper5
	9/24/18 
	0.63

(ND-630)
	ug/L
	1300
	Corrosion of household plumbing


(
Notes:

1–A level I assessment was performed in response to the positive sample results collected from the Dennison Road Tank.  A Level I assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system.  Based on the assessment it was determined that the potential cause of the positive samples may have been a combination of the temperature of the water and a low chlorine residual.  Chlorine residuals were increased and the system was thoroughly flushed in response to the positive results. 
Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system.  We found coliforms indicating the need to look for potential problems in water treatment or distribution.

Notes (cont’d):

2 – Stage 2 TTHM and HAA samples were collected quarterly.  This represents the highest running annual quarterly average calculated from data collected.  
 3– Some people who drink water containing trihalomethanes in excess of the MCL over many years may experience problems with their liver, kidneys, or central nervous systems, and may have an increased risk of getting cancer.
 4– Lead and Copper: (2018 Survey) 90% of samples must be less than the Action Level (AL) 90th percentile.  The City of Rochester collected 63 samples.  Six out of 63 samples tested exceeded the Lead AL.  Zero out of 63 samples exceeded the Copper AL. In 2018, 5 of the 63 samples were collected from homes in the LCWSA Consolidated System and the Village of Livonia. None of those samples exceeded the AL for lead or copper.
Definitions:
Maximum Contaminant Level  (MCL): The highest level of a contaminant that is allowed in drinking water.  MCLs are set as close to the MCLGs as feasible.

Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected risk to health.  MCLGs allow for a margin of safety.

Maximum Residual Disinfectant Level (MRDL): The highest level of a disinfectant allowed in drinking water.  There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants.
Action Level  (AL): The concentration of a contaminant, which if exceeded, triggers treatment or other requirements, which a water system must follow.
Treatment Technique (TT): A required process intended to reduce the level of a contaminant in drinking water.

Non-Detects (ND): Laboratory analysis indicates that the constituent is not present.

Milligrams per liter (mg/l): Corresponds to one part of liquid in one million parts of liquid (parts per million - ppm).
Micrograms per liter (ug/l): Corresponds to one part of liquid in one billion parts of liquid (parts per billion - ppb).

Total Trihalomethanes(TTHM): means the sum of the concentration of chloroform, bromodichloromethane, dibromochloromethane and bromoform
Haloacetic Acids (HAA):  means the sum of the concentrations of five specific haloacetic acid compounds:
 (mono-,di- and trichloracetic acid, and mono- and di-bromoacetic acid) 
What does this information mean?
The table shows that our system uncovered some problems this year.  The MCLs for total coliform contaminants were exceeded at the Dennison Rd Tank from 8/15-19 to 8/2/19 resulting in increasing chlorine levels in the system, additional flushing and re-sampling.  In addition, the MCL for TTHMs was exceeded in the 1st, 2nd 3rd and 4th quarter of 2019 resulting in public notification and corrective action. Both incidents were described previously in this report. 

Is our water system meeting ALL rules that govern operations? During  2019 our system was in compliance with applicable State drinking water operation, monitoring and reporting requirements. 
We are required to provide the following information regarding lead in drinking water:
If present, elevated levels of lead can cause serious health problems, especially for pregnant women, infants, and young children.  It is possible that lead levels at your home may be higher than at other homes in the community as a result of materials used in your home's plumbing.  The LCWSA, along with The City of Rochester Water Treatment Plant, is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components.  When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using it for drinking or cooking.  If you are concerned about lead in your drinking water, you may wish to have your water tested.  Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline (1-800-426-4791) or at www.epa.gov/your-drinking-water/basic-information-about-lead-drinking-water.

Do I Need to Take Special Precautions? tc \l4 "Do I Need to Take Special Precautions? 
Some people may be more vulnerable to disease causing microorganisms or pathogens in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections.  These people should seek advice from their health care provider about their drinking water.  EPA/CDC guidelines on appropriate means to lessen the risk of infection by Cryptosporidium, Giardia and other microbial pathogens are available from the Safe Drinking Water Hotline (800-426-4791).  
Information for Non-English Speaking Residents 
This report contains important information about your drinking water. Translate it, or speak with someone who understands it.
Spanish
Este informe contiene información muy importante sobre su agua beber.  Tradúzcalo ó hable con alguien que lo entienda bien. 
HOW CAN I SAVE MONEY ON WATER?

Although our system has an adequate amount of water to meet present and future demands, there are a number of reasons why it is important to conserve water:

· Saving water saves energy and some of the costs associated with both of these necessities of life;

· Saving water reduces the cost of energy required to pump water and the need to construct costly new wells, pumping systems and water towers; and

· Saving water lessens the strain on the water system during a dry spell or drought, helping to avoid severe water use restrictions so that essential fire fighting needs are met.

You can play a role in conserving water by becoming conscious of the amount of water your household is using, and by looking for ways to use less whenever you can.  It is not hard to conserve water.  Conservation tips include:

· Automatic dishwashers use 15 gallons for every cycle, regardless of how many dishes are loaded.  So get a run for your money and load it to capacity.

· Turn off the tap when brushing your teeth.

· Check every faucet in your home for leaks.  Just a slow drip can waste 15 to 20 gallons a day.  Fix it and you can save almost 6,000 gallons per year.

· Check your toilets for leaks by putting a few drops of food coloring in the tank, watch for a few minutes to see if the color shows up in the bowl.  It is not uncommon to lose up to 100 gallons a day from one of these otherwise invisible toilet leaks.  Fix it and you save more than 30,000 gallons a year.

· Use your water meter to detect hidden leaks.  Simply turn off all taps and water using appliances, then check the meter after 15 minutes.  If it moved, you have a leak.

For more information, log on to http://www.dec.ny.gov/lands/5009.html.
System Improvementstc \l5 "System Improvements
The LCWSA operates & maintains the Village of Livonia district via an agreement with the Village of Livonia.  The remainder of the service areas are leased/owned and operated by the LCWSA.  The following improvements were completed in 2019:
· Numerous curb boxes were replaced;
· Chlorine analyzers were maintained on a semi-annual basis by HACH Instruments;          

· At least four (4) major leaks were discovered and repaired, along with eight (8) service lines that were repaired.
· Backflow Prevention program implemented for LCWSA customers.

· Water Main Extension from Conesus to the Groveland Correctional Facility was completed and put in operation.
The following projects are in-progress and/or scheduled for completion in 2020:

· Leak detection will be on-going;

· Hydrant and Valve maintenance will take place in all service areas;

· Curb box replacements will take place in all service areas;
· Meter replacement program on going;
· A project to get higher pressure to the water customers on Shelly Rd. and Meadow Dr. along with the Livonia High school campus.

BULK WATER AVALIBILITY
Water fill stations are located at the Town of Livonia Highway Department (50 Commercial St) and at the Town of Sparta Community Center (7351 Route 256). 
Closingtc \l5 "Closing
Thank you for allowing us to continue providing your family with clean, quality water this year.  In order to maintain a safe and dependable water supply, additional improvements may be necessary in the future.  We will keep you informed of any significant changes in services.  Please call LCWSA at (585) 346-3523 if you have any questions. In case of an Emergency, please call the LCWSA at (585) 346-3523.  Questions regarding water billing should be directed to the LCWSA at (585) 346-3523 between the hours of 8:00am and 4:00pm (Monday – Friday).
                                             Annual Drinking Water Quality Report for 2019

Town of Geneseo Water District #3 and District #2 North

(Public Water Supply ID# NY2530023)

Water District #3 serves Town of Geneseo customers with service connections along Lakeville Road (Route 20A) west of West Lake Road (Route 256) and east of Route 390, including Lakeville Village Estates.

Water District #2 North (2N) serves Town of Geneseo customers with service connections along West Lake Road (Route 256) from Reservoir Road north to Gray Shores, including Blue Heron Drive.  In 2020, this water district will be served by water purchased from the Village of Geneseo from Conesus Lake as the source beginning on May 18, 2020.
WHERE DOES OUR WATER COME FROM?

Water consumed by the Town of Geneseo Water District 3 and District 2 North is purchased from Livingston County Water and Sewer Authority (LCWSA).  In 2019, the Town of Geneseo Districts 2N and 3 had 190 active service connections and served water to a population of 482. The total amount of water purchased was 15,238,700 gallons. As part of routine maintenance, the Town of Geneseo flushed approximately 138,600 gallons of water.  The Town Water District rate was $4.50 per 1,000 gallons plus a $30.00 per quarter water service fee.   

DO THE WATER DISTRICTS TEST OUR WATER?

The Town of Geneseo Water District also routinely monitors the drinking water for Total Coliform, Asbestos, and Disinfection Byproducts, including Total Trihalomethanes (TTHM) and Haloacetic Acids (HAA5) in compliance with State and Federal drinking water standards.  In 2019, the Town took 12 “at the tap” samples for the presence of coliform bacteria.  Of these samples, 1 tested positive for coliform bacteria.  However, repeat confirmation samples tested negative for coliform bacteria.  The annual TTHM and HAA5 samples were collected in August with results illustrated in the table below.  All samples complied with State and Federal drinking water standards.

	Town of Geneseo, District 3 

	Contaminant
	Violation

Yes/No
	Date of Sample
	Level Detected
	Unit of Measure
	MCLG 
	Regulatory Limit (MCL, MRDL, TT)
	Likely Source of Contamination

	Chlorine Residuals Measured in Distribution 

	Chlorine Residual
	No
	Monthly
	Range

0.03–0.14
	mg/l
	N/A
	4.0
	Water additive used to control microbes

	Stage 2 Disinfection Byproducts 

	Total Tri-halomethanes

(TTHMs)
	No
	8/06/191
	70
	ug/L
	N/A
	80
	By-product of drinking water chlorination needed to kill harmful organisms.  TTHMs are formed when source water contains large amounts of organic matter.

	Haloacetic

Acids (HAA5)
	No
	8/06/191
	21
	ug/L
	N/A
	60
	By-product of drinking water

Disinfection.

	Inorganics

	Asbestos
	No
	10/16/18 3
	2
	MFL
	7
	7
	Decay of asbestos cement water mains and erosion of natural deposits

	Microbiological Contaminants

	Total Coliform Bacteria
	No2
	09/04/19
	Positive2
	N/A
	0
	2 or more positive samples 
	Naturally present in the environment


Notes:
1 - One sample of water was collected and analyzed for TTHM and HAA5.  Samples must be collected from a point in distribution that represents the maximum residence time and during the month with the highest water temperature.  
2 – Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system.  We found coliforms indicating the need to look for potential problems in water distribution.  No Treatment Technique (TT) violation occurred because the required 3 repeat confirmation samples were negative for Total Coliform (TC) bacteria.  

3 – Some data, though representative, are more than a year old.  Asbestos sampling is done every 9 years.  

DEFINITIONS
Maximum Contaminant Level (MCL) means the maximum permissible level of a contaminant in water, which is delivered to any user of a public water system.

Maximum Contaminant Level Goal (MCLG) means the level of contaminant in drinking water below which there is no known or expected risk to health.

Treatment Technique (TT) means a required process intended to reduce the level of a contaminant in drinking water.

Total Trihalomethane (TTHM) means the sum of the concentration of trichloromethane (chloroform), dibromochloromethane, bromodichloromethane and tribromomethane (bromoform).

Haloacetic acids (five) (HAA5) mean the sum of the concentrations in milligrams per liter of five specific haloacetic acid compounds.

Million Fibers per Liter (MFL) is a measure of the presence of asbestos fibers that are longer than 10 micrometers.

Maximum Residual Disinfection Level (MRDL) is the level of disinfectant added for water treatment that may not be exceeded at the customer’s tap without an unacceptable possibility of adverse health effects.

Milligrams per Liter (mg/l) corresponds to one part of liquid in one million parts of liquid (parts per million – ppm)

Micrograms per Liter (ug/l) corresponds to one part of liquid in one billion parts of liquid (parts per billion – ppb)

IS OUR WATER SYSTEM MEETING OTHER RULES THAT GOVERN OPERATIONS?

In 2019, the Town of Geneseo Water Districts 3 and 2 North were in compliance with applicable state drinking water operating, monitoring and reporting requirements.
BULK WATER AVAILABILITY:

The Town operates a water tank fill station at the storage tank site on Burbank Drive.  This water is purchased from the Village of Geneseo Public Water Supply.  This is a coin operated “water-salesman”.  Currently rates for this bulk water are $10.00 per 1000 gallons ($.25 per 25 gallons).

Need More Information?

The Town of Geneseo Water Department can be reached at (585)243-1544 Monday through Thursday between 6AM and 4:30PM, for any questions regarding water service. In case of Emergency please call the Livingston County Water & Sewer Authority at (585)346-3523. Questions regarding water billing should be directed to the Livingston County Water & Sewer Authority at (585)346-3523 between the hours of 8:00AM and 4:00 PM.  

